- Contents

Notation	2

1	Introduction	2

1.1	Theory of condition monitoring	2

1.2	Overview of this thesis	4

2	Feature Extraction	5

2.1	Mel Frequency Cepstral Coefficients	6

2.2	Multi Fractal Dimensions	7

2.3	Kurtosis	8

3	Semi-supervised classification	9

3.1	Support Vector Data Description	12

3.2	K-Center	16

3.3	K-Means Data Description	17

3.4	Nearest Neighbor Data Description	17

3.5	Parzen Window Data Description	18

3.6	Self Organizing Maps	18

3.7	Random forest	20

3.7.1	Variable importance	21

3.7.2	Proximity	21

3.7.3	Class prototypes	22

3.7.4	Outlier measure	22

3.7.5	Unsupervised training	22

3.7.6	Semi-supervised classification	23

4	Experiments	23

4.1	Roller Bearing Datasets	25

4.2	Feature Extraction	27

4.2.1	Mel Frequency Cepstral Coefficients	27

4.2.2	Higuchi Fractal Dimensions	28

4.3	Random Forest Data Description	29

4.4	Classification and Evaluation	31

4.4.1	Performance evaluation	31

4.4.2	Feature reduction experiment	33

5	Conclusions	34

6	References	35

# Notation

# Introduction

## Theory of condition monitoring

Condition Monitoring is the monitoring of a system “by studying certain selected parameters in such a way, that significant changes of those parameters are related to a developing failure” (Marwala, 2012). In manufacturing, condition monitoring can help to reduce operation costs significantly by preventing machine failure and supporting a condition based maintenance approach (Long). In other areas such as aerospace or construction, condition Monitoring can play a crucial role in the prevention of severe accidents.

Figure 1 illustrates a general condition monitoring framework (Marwala, 2012).

Fig. 1  Condition Monitoring Framework

A data acquisition devise comprises sensors measuring significant machine parameters and devices such as D/A-converters, which transform the acquired data into a signal suitable for further processing.  Typical examples of sensors include accelerometers, thermometers or strain gauges. The outputs of data acquisition devices are usually discrete time signals, which can be represented by a set of tuples of the form

SN=y1,t1,y2,t2,…,yN,tN

Where N denotes the number of pairs in the Signal sequence and yi represents a sensor value such as the amplitude recorded at time ti.

Purpose of the data analysis step is the extraction of suitable attribute-value pairs (features) from time signals, which can be used as inputs to decision algorithms. Data analysis techniques can roughly be grouped into three categories:

- Time domain methods, measuring aspects of the raw time signal directly (e.g. kurtosis or fractal dimensions, )

- Frequency domain transformation (Fourier transform)

- Time-frequency domain transformations, such as the wavelet transform, the short time Fourier transform or the Cepstrum analysis

Outputs of the Data Analysis step are usually feature vectors of the form

xn=(x1,x2,…,xD)

where n denotes a specific data vector in a sequence of N vectors, x1,…,xD are the particular feature values (kurtosis, fractal dimension etc.), and  D is the feature set dimension.

The number of features D, can have a significant effect on the performance of classification algorithms, of which many are susceptible to the curse of dimensionality (Bishop, 2009). Several feature reduction techniques exist, to select the features which are best suited to distinguish machine conditions. By far the most popular of these methods is the principal component analysis.

A decision making device can be any method capable of estimating machine conditions from feature vectors. The available methods include statistical algorithms, machine learning and pattern recognition techniques or combinations of several methods, also known as Batch or Boosting algorithms (Marwala, 2012).

An important criterion for the selection of suitable decision techniques is the availability of data samples of the form

xn,yn

Where xn is the nth feature vector in a set of N sample vectors and yn is the label of a known machine condition applied to xn. The generation of sample data sets typically requires the involvement of a supervisor, with the expert knowledge to apply meaningful labels to experimental sample data. Techniques utilizing such datasets to construct a model of machine conditions are therefore also known as supervised methods. The term supervised usually refers to sample data sets containing representative data samples for all known machine conditions. If sample data is available for only one condition, semi-supervised techniques can be used to train models which are able to distinguish the known condition from all other possible conditions. Semi-supervised scenarios typically occur if it’s either too expensive or impossible to obtain sample data covering all possible conditions. In the condition monitoring of machines, data representing normal conditions can often be obtained easily, whereas the acquisition of data representing fault conditions requires the destruction of a machine in all possible ways (Myers, Japkowicz, & Gluck, 1995). In such scenarios, the normal samples can be used to train semi-supervised models which are able to detect anomalies during machine runtime. This is also known as outlier or novelty detection (Banerjee, Chandola, & Kumar, 2007).

If no sample data is available, unsupervised methods can be used to detect meaningful patterns in a datasets. Unsupervised methods are.

## Overview of this thesis

Roller bearing data has already been used in several publications, to evaluate and proof condition monitoring concepts (Marwala, 2012), (Nelvamondo, Marwala, & Mahola, 2006), (Li, Chow, Tipsuwan, & Hung, 2000). Most of these papers used both normal and outlier training data, to evaluate the efficiency of supervised classifiers such as Neural Networks or Support Vector Machines. Goal of this thesis is the design and evaluation of an exemplary semi-supervised condition monitoring approach, using the roller bearing data collection from (Case Western University) as benchmark.

My contribution involves

- Research in the field of condition monitoring and concise presentation of a general condition monitoring approach (1.1)

- Selection of a suitable benchmark data set (4.1)

- Research in feature extraction methods and selection of suitable methods with respect to the benchmark data set (2,4.2)

- Implementation of a Higuchi fractal dimension (HFD) feature extraction method (4.2.2)

- Research in the field of  one-class-classification and selection of suitable classifiers in relation with the benchmark data set and the available features (3,4)

- Design and implementation of a one-class-classifier based on random forest

- Implementation and evaluation of a complete semi-supervised condition monitoring approach, including feature extraction, feature selection, classification and tests (4)

The first part of this thesis introduces and discusses several techniques, starting with feature extraction methods in chapter 2. Chapter 3 provides a general approach for the construction of One-Class-Classifiers and continues with an introduction of various semi-supervised classifiers which can be found in literature. The final part of this chapter discusses several aspects of the random forest classifier (3.7), of which a semi-supervised version was developed and evaluated as part of this thesis (3.7.6), (4.3).

The techniques introduced in the first part, are evaluated in the second part of this thesis (4), which starts with a description of the roller bearing data set in 4.1. The feature extraction results are briefly discussed in 4.2 and 4.3 provides a proof of concept of the semi-supervised random forest version introduced in 3.7.6. Two experiments where designed to evaluate the complete outlier detection scenario, of which the first one (4.4.1) compares the classifier performances, while the second analyzes applicability of random forest for feature reduction purposes.

# Feature Extraction

Analysis of vibration data measured during machine runtime is the most popular approach to Machine Condition Monitoring (Nelvamondo, Marwala, & Mahola, 2006). Consequently, many different methods have been applied to analyze machine vibration signal. Some of these methods, such as the peak level, crest factor or kurtosis analysis use the time signal directly to measure certain aspects of the data. Other methods, such as the Fourier or the Wavelet Transform, transform the time signal into another domain to expose meaningful data aspects hidden in the time signal.

In this thesis, a mixture of several techniques is used to extract features that can be used to distinguish between normal and faulty machine conditions. Two of these techniques, kurtosis and Multi Fractal Dimensions  process the time signal directly. The third method, Mel Frequency Cepstral Coefficients transforms the time signal into a time-frequency-domain to extract features.

## Mel Frequency Cepstral Coefficients

Mel Frequency Cepstral Analysis is a technique commonly used in Speech Recognition, which can capture the dynamic characteristics of a signal by extracting both linear- and non-linear features (Marwala, 2012) .

The Cepstrum C(τ) is defined as the inverse Fourier Transform of a logarithmic Frequency Spectrum

Cτ=F-1log⁡(Ff

(2.1.1)

Where F-1 is the inverse Fourier Transform and Ff is the frequency spectrum of a signal.

Some insight into the general properties of a Cepstrum can be gained by regarding a time signal   y(t) as the output of a Linear Time Invariant (LTI) system, characterized by its impulse response h(t). The output y(t)  is then given by the convolution of the input signal xt and the impulse response

yt=xt*ht.

(2.1.2)

In the Frequency domain, the convolution in (2.1.2) is transformed into the multiplication

Yf=XfHf.

(2.1.3)

With (2.1.1) and (2.1.3) the cepstrum of an LTI gives

Cτ=F-1log⁡(XfHf

Cτ=F-1log⁡(Xf+log⁡(Hf

Cτ=F-1log⁡(Xf+F-1log⁡(Hf

Cyτ=Cxτ+CHτ

(2.1.4)

From (2.1.1) and (2.1.4) it can be seen, that a Cepstrum separates the input signal and the impulse response in the time domain, which can be very useful when time signals yt are compared which are generated by different input signals xt in combination with the same system characterized by ht.

Applied to the Condition Monitoring of machines, yt in (4.1.2) represents the signal measured by accelerometers at a certain point outside the machine. The actual information about the machine condition is contained in the signal xt which is generated somewhere inside the machine and filtered by the transmission path in the machine. The Cepstrum separates the condition signal from the transmission path (Kolerus & Wassermann, 2008).

The Mel Frequency Coefficients used in various Condition Monitoring applications (Nelvamondo, Marwala, & Mahola, 2006), (Marwala, 2012) are based on a Mel-transformation of the Frequency Spectrum and can be calculated by

- Transforming the input signal xt from the time domain to the time-frequency domain through a windowed  Fast Fourier Transform:

ym=1Fn=0F-1x(n)w(n)e-j2πFnm

Where F is the number of windows,  x(n)  is the time signal in window n and  w(n) is the Hamming Window given by

wn=β(0.5-0.5cos2πnF-1)

with 0≤n≤F-1 and the normalization factor β.

- Changing the frequency spectrum to the Mel scale through the equation

mel=2595×log101+fHz700

- Converting the logarithmic Mel Spectrum back to the time domain with the Discrete Cosine Transform

Cmi=n=0F-1cos⁡mπFn+0.5log10(Hn)

## Multi Fractal Dimensions

Fractals are patterns consisting of sub-patterns which are equal or similar in shape to the complete pattern. A fractal shape is characterized by a Fractal Dimension which exceeds the topological dimension of the shape and may fall between two integer numbers (Mandelbrot, 2004).

Applied to the analysis of time series, Fractal Dimension can be seen as a measurement for the irregularity of a curve (Polychronaki, et al., 2010), which has a topological dimension of one and a Fractal Dimension between one and two. Various algorithms exist for the calculation of the fractal dimension of time series, often based on some kind of length or distance measurements between the discrete points of a curve over several scales. Popular algorithms involve the Box-counting Method, Katz’s Method and Higuchi’s Method (Raghavendra & Dutt, 2010).

Fractal Dimension Measurement has been successfully used for the extraction of features from machine-vibration data (Nelvamondo, Marwala, & Mahola, 2006), (Marwala, 2012). In this thesis, Higuchi’s method is used .Given a discrete time series consisting of N samples 1,y2,…,y(N) , the Higuchi Fractal Dimension (HFD) is calculated by (Esteller, Vachtsevanos, Echauz, & Litt, 2001)

- Constructing k new time series as

xmk=xm,xm+k,xm+2k,…,xm+N-mkk,

for m=1,2…,k

where m indicates the initial time value, k indicates the discrete time interval between points and a is the next lowest integer value to a.

- Computing the average length for each of the new time series xmk as

Lmk=i=1N-mkxm+ik-xm+i-1k(N-1)N-mkk

for several scaling factors ranging from 1 to a freely chosen kmax , with m=1,2…,k and the normalization factor

(N-1)N-mkk.

- Calculating the complete length as sum of the average lengths:

Lk=m=1kLmk

- Estimating the Fractal Dimension D as the slope of the least squares linear best fit line for  the curve lnL(k) versus ln1k , since Lk~k-D.

## Kurtosis

Kurtosis is a general term for any measure of curve Peakedness. In statistics, Kurtosis is defined as the normalized fourth-order moment, which can be calculated by

K=1Ni=1Nxi-μ4σ4

Where N is the number of samples and the normalization factor is the squared second order moment, σ4.

Kurtosis as quantification of peak sharpness can be useful in Condition monitoring of machines, where faults often manifest themselves in significant changes in the sharpness or spiking of the vibration signal (Nelvamondo, Marwala, & Mahola, 2006).

# Semi-supervised classification

A classifier can be represented by a function

y=f(x,w)

Where x is the data vector to be classified,  w is a vector of parameters which are inferred from labeled samples (training data set) and y is the estimated class for x.

The optimization of the parameters w is often done by minimizing some kind of error function such as the Sum of Squares Error, which is given by

Ew=12n=1Nfxn,w-yn2

where yn is the class label of the nth training sample vector xn.

In a supervised setting, where training samples are available for all possible classes, a classifier creates a classification boundary separating objects belonging to different classes. Figure 2 illustrates a (non-optimal) class boundary for a two-class problem with the two features x1 and x2.

Fig. 2: Two-Class-Classification

The quality of a trained classifier is measured by its ability to correctly classify unseen data which was not used in the training of the classifier. This important property, also known as generalization (Bishop, 2009), depends on a number of factors. A fundamental aspect for the performance of a supervised classifier is the quality of the sample set used for training. If the samples do not represent the underlying population well, the classifier will perform poorly on unseen samples. Other important aspects are the choice of classifiers and the degree of adaption to the sample set during training, which is controlled by the number of parameters. The line illustrated in figure 2 is not flexible enough to capture a class distribution which is not linearly separable and thus always shows a bias towards a certain class distribution. If a classifier is too flexible on the other hand, the adaption to all the details in the training set (including the noise) may be too high, which is also known as overfitting and results in poor generalization. The problem of finding an optimal degree of complexity between these two extremes is often referred to as bias-variance trade-off (Domingos, 2012).

If sample data is available for only one of the possible classes, a different classification approach has to be chosen. Since nothing is known about the distribution of classes for which no sample data is available, the general goal in such a semi-supervised case is to find an optimal description for the known target class. New data can then be compared to this data description and accepted as target class member if it matches the model closely enough, or rejected as outlier otherwise.  Figure 3 illustrates this concept with a closed boundary as data description of the target training objects.

Fig. 3: One-Class-Classifier

This type of classifier can also be represented by a mapping of the form

f:RD→{1,-1}

Were label 1 is applied to data objects which are estimated to belong to the target class and the label -1 to all others. Depending on the context, this classification approach is also known as Data Description, Novelty Detection, Outlier Detection or Concept learning. A comprehensive survey of One-Class-Classifiers can be found in (Tax M. J., 2001).  A

All One-Class-Classifiers applied in this thesis consist of two main components. The first component is a function measuring the relation of a new data objects to the target class. This function can be either some kind of a distance measure d(x) or a resemblance probability p(x). The second component is a threshold θ defining the decision boundary.  New objects are accepted when their distance to the target dataset is smaller than the threshold:

fx=Idx<θ

(3.1)

or when their probability to belong to the target dataset is above the threshold:

fx=I(px>θ)

(3.2)

Where I(∙) is the indicator function.

A common choice for the distance function is the Euclidean Distance.

For a distance based method, the threshold can be defined by

FT+=1N1NIdxi<θ

(3.3)

where xi are the training samples and FT+ is the fraction of accepted target samples from the training data. For a probability based approach, the threshold is accordingly given by

FT+=1N1NIpxi>θ

(3.4)

The optimal value FT+ has to be found experimentally. For some robustness against outliers in the dataset, FT+ should be smaller than one. However, the smaller  FT+ , the larger the classification error on the target objects.

For the evaluation of the One-Class-Classifier performance, two types of errors have to be considered. Error Type I is defined as

E1=FT-

where FT- is the fraction of rejected targets, and Error Type II is defined as

E2=FO+

Where FO+ is the number of accepted outliers.

The One-Class-Classificers used in this thesis, are briefly introduced in the following chapters.

## Support Vector Data Description

For a two-class problem with two features, a Support Vector Machine finds an optimal class boundary by identifying the points closest to the class boundary and maximizing their perpendicular distance (also known as margin) to the class boundary. This principle is illustrated in figure for a linearly separable two-class training set.

Fig. 4: Support Vector Machine

The Support Vectors, marked by circles in figure 4 define the optimal decision boundary (in this case a simple line) from both sides. Generalized two D dimensions, the decision boundary is a hyperplane of the form

yx=wTx+b

(3.1.1)

where for all points lying on the plane  yx=0.

The optimal values for the parameters w and b can be found by solving (Bishop, 2009)

argmaxw,b1wminntnwTxn+b.

(3.1.2)

If (3.1.1) is set to

wTxn+b=1

(3.1.3)

for the n training data points closest to the hyperplane, then

wTxn+b≥1

(3.1.3)

for all training data points and  (3.1.2) can be reduced to

arg maxw,b1w

which is equivalent to minimizing

arg minw,b12w2.

(3.1.4)

subject to the constraints given by (3.1.3).

With the introduction of one Lagrange Multiplier an for each constraint defined by (3.1.3), the Lagrangian is given by

Lw,b,a=12w2-n=1NantnwTxn+b-1.

Optimizing the Lagrangian with respect to w,b and a, plugging the results of the optimization into (3.1.1) and with the introduction of a kernel function, the Support Vector classifier can be formulated as

fx=n=1Nantnk(x,xn)+b

(3.1.5)

Where training vectors xn corresponding to an>0 are the support vectors and k(x,xn) is a kernel function (Bishop, 2009). The class membership of an unknown test point x is indicated by the sign of (3.1.5).

According to the general definition of One-Class-Classifiers given above, the semi-supervised variant of a Support Vector Machine - also known as Support Vector Data Description (SVDD) – tries to find a boundary enclosing all the target training samples. This boundary is a hypersphere with radius R and center a, as illustrated in figur  (Tax & Duin, 2004).

Fig. 5: Support Vector Data Description

The fundamental goal in SVDD is the minimization of the radius

arg minR R2

such that the hypersphere contains all training data points, which is expressed by the constraints

xn-a2≤R2

(3.1.5)

Where a and R are the center and the radius of the hyperspere and xn is the n-th training data vector.

To deal with the occurrence of outliers in the target training data set as indicated by the point outside the boundary in figure 5, the constraints in (3.1.5) have to be extended by so-called slack-variables ξn to

xn-a2≤R2+ξn.

(3.1.6)

With the new constraints the optimization problem changes into

arg minR,ξn=1Nξn+R2

(3.1.7)

where ξn≥0 and C controls the tradeoff between a tight decision boundary and the influence of outliers.

A compact formulation of the optimization problem is obtained by combining the objective function(3.1.7) with the corresponding constraints (3.1.6) to the Lagrangian

LR,b,ξ,a, α,μ=R2+Cn=1Nξn-n=1NanR2+ξn-xn2-2xna+a2-n=1Nμnξn.

(3.1.8)

Where α and μ are the Lagrange Multipliers.

By a sequence of steps (Tax M. J., 2001), the final SVDD classifier can be derived as

fx,α,R=Ix-a2≤R2=Ix2-2n=1Nanx*xn+n,manamxn*xm≤R2

(3.1.9)

Where x is the test vector to be classified,  xn and xm are the nth and the mth training vectors and  an, am are Lagrange multipliers which are only nonzero, if they correspond to support vectors  xn or  xm.

The vector products in (3.1.9) can again be replaced by kernels, resulting in the classifier

fx,α,R=Ix-a2≤R2=Ix2-2n=1Nankx,xn+n,manamkxn,xm≤R2

Figures 4 to 6 show SVDD boundaries with a Radial Basis Function Kernel and different values for sigma, fitted to the same banana shaped target data set.

Fig. 6 SVDD with RBF, sigma=2

Fig. 7, SVDD with RBF, sigma=3

Fig. 8, SVDD with RBF, sigma=5

## K-Center

K-Centers is based on the Domain Approximation algorithm (Ypma & Duin, 1998), where a subset Sk of k support objects is selected from the N samples of a training dataset X and each of the support objects is surrounded by a receptive field of equal radius r. A sample xi of the dataset X lies in the receptive field of a support object sp if

p=argminjdxi,sj

And

d(xj,sj)

where d can be any kind of distance measure.

The optimization problem of finding a optimal subset Sk with a minimal radius r such that all data samples in X are covered by the receptive field of one support object, can be solved with a variant of the k-means algorithm known as k-centers. K-centers finds a subset Sk such that the maximum distance of all minimum distances between dataset objects and support objects is minimized (Tax M. J., 2001). Thus, the goal of K-Centers optimization is the minimization of the error function

εk-center=maximinkxi-sk2.

With the trained support objects sk, the distance of a test object x to the target set can be calculated by

dk-centersx=minkx-sk2.

## K-Means Data Description

K-Means is a clustering algorithm similar to k-centers where k prototypes μk are defined and the clustering is based on a distance measure between training data objects and their prototypes. The main differences to k-centers are the placement of the prototypes - which in k-Means is not constrained to coincide with existing training data objects - and the error function, which is defined by

εk-center=iNminkxi-μk2.

(5.4.1)

In k-Means (Tax M. J., 2001).

The minimization of the Error (5.4.1) implies a placement of the k prototypes such that the distance of each training object to the closest prototype is a minimum. A typical algorithm for this optimization problem starts with a random placement of k prototypes and calculates k clusters containing the training data objects closest to the prototype μk. Then for each cluster, a new prototype is calculated by simply taking the mean of the data points in that cluster. This procedure is reiterated until the prototypes are stable.

After training of the prototypes, new objects can be compared to the target class description by simply calculating their Euclidean Distance to the closest prototype. The classifier accepts new objects if the distance to their prototype is below the threshold value and rejects them otherwise:

fx=Iminkx-μk2≤θ

## Nearest Neighbor Data Description

K-Nearest-Neighbor classification is based on a local density estimation of the form (Bishop, 2009)

px=KNV

Where K is some fixed integer value, N is the size of the training dataset and  V(x) is the volume of a small sphere centered on a data vector x. In the supervised case, a test value x is classified by increasing V until the sphere contains exactly K data points and assigning the majority class of all data points in the sphere to x.

In an adaption of the K-Nearest-Neighbor method to semi-supervised problems (Tax M. J., 2001), the distance of a test object x to its nearest neighbor in the training set NNx is compared to the distance between the nearest neighbor of x and the nearest neighbor of the nearest neighbor of x,  NNNNx. The test object is accepted if the distance dx,NNx is smaller than or equal to the distance dNNx,NNNNx. This can be formalized by the classifier function

fx=Ix-NNxNNx-NNNNx≤1

## Parzen Window Data Description

In its general form, a Parzen Window estimator is given by (Bishop, 2009)

px=1Nn=1N1hDkx-xnh

(5.7.1)

where xn is the nth training point, h is the kernel width and kx-xnh is a kernel function which has to satisfy the constraints

k(u)≥0

And

k(u)du=1.

With a Gaussian kernel, The Parzen Windows estimator is given by

px=1Nn=1N12πh2D2exp-x-xn22h2.

Where h is the standard deviation of the Gaussian components.

Parzen Window can be used directly for One Class Classification. The only parameter h is trained through a maximum likelihood method (Tax M. J., 2001).

## Self Organizing Maps

A Self Organizing Map consists of several nodes or neurons. Each node has a defined position in an N-dimensional grid structure and is associated with a D-dimensional prototype vector, representing D-dimensional training data objects.   N is often chosen to be 1 or 2, such that

N≪D.

The position of a neuron in the N-dimensional grid – or feature space – reflects the neighborhood relations in the D-dimensional data space, such that neighboring nodes in the grid are associated with neighboring prototypes in data space. Through this topology preserving property, SOMs create a mapping from a high dimensional data space to a low dimensional feature space, and allow the visualization of relations from a high dimensional data space (van der Heijden, Duin, de Ridder, & Tax, 2004).

The training starts with an initialization of the prototypes, e.g. by randomly assigning a training data object to each node. Then, for each training data object xn the following two steps are repeated:

- Detection of the node ωki  whose associated prototype is closest to the data object  xn by calculating

kxn=argminjxn-ωji

Where the subscript j denotes the number of the node and the superscript i specifies the iteration step in the training sequence.

- Updating of the winning node and its grid neighbors by computing for each node j

ωj(i+1)=ωji+ηihikxn-jxn-ωji

Where ηi is a learning rate which can be set to decline with the iteration count and hi is a weighting function which determines by how much a node is updated. The input to this weighting function kxn-j is the distance between the winning neuron determined in step one and neuron ωji which is to be updated. The weighting function has to satisfy the constraints

hi0=1

and

hi∙≤1

to ensure that the winning node receives the largest update.

A common choice for the weighting function is the Gaussian function

hix=exp⁡-x2σ(i)2

Where σ(i)2 defines the width of the neighborhood area, which can decrease with each iteration step.

Other choices that have to be made prior to the training are the structure and dimension of the grid and the number of neurons. Often, a 2-dimensional grid is chosen for visualization purposes. In many cases the best combination of parameters has to be found experimentally (van der Heijden, Duin, de Ridder, & Tax, 2004).

To classify a test object, its distance to the closest prototype is measured. If this distance is below the predefined threshold, the test object is accepted as member of the target class and rejected otherwise.

## Random forest

Random Forests (Breiman , Random Forests, 2001) is an ensemble learning method for classification and regression which grows many trees from sampled subsets of a training set. New objects are run down all trees and labeled with the class that a majority of the trees voted for.

Each tree in the forest is grown in the following way:

- If the number of cases in the original training set is N, N cases are sampled from the training set, with replacement.

- If the number of attributes in the training set is M, a number m≪M is chosen. For each node in the tree m attributes are randomly sampled from the original M attributes and the best split among these m attributes is used to calculate the split for the node.

- Each tree is grown to maximum size, without pruning.

The individual trees are grown using the CART algorithm with an additional randomization moment in the attribute selection at each node as described in step 2 above. CART is a decisions tree algorithm which uses the gini criterion to split nodes and only allows binary splits (Quinlan & Kohavi , 1999). The split decision is made by selecting at each node the attribute m resulting in the highest information gain

GS,m=IS-i=1tSiSISi

(5.2.1)

Where S is the dataset before splitting, Si are the splitted datasets and IS is the gini index

IS=1-j=1xRFCj,S2

(5.2.2)

where RFCj,S is the relative frequency of cases with class j in dataset S.

Random Forests have a number of useful features, making them a good choice for a variety of classification and regression problems. Among the advantages of random forests are

- A higher accuracy than many other algorithms

- The abiltiy to run on large datasets

- The ability to process thousands of attributes without the need of feature selection

- An implicitely calculabe generalization error without the need of methods such as Cross Validation

- The capability to calculate a proximity between cases, which can be used to cluster date or to detect outliers

- Efficient methods for replacement of missing data

- The ability to measure attribute importance, which can help to reduce the datasets for other applications

- A posibility to find class prototypes based on the proximity measure, which can help to gain more insight into the data

- Extensibility to unlabeled data through the two previous features

Some of the features are a result of the basic tree growing process, while others are based on optional computations and data structures which can easily be integrated into the tree construction.

For the construction of each tree in a random forest, a training set of N cases is uniformly sampled from the original training set with replacement. Thus, a certain number of training number is left out in the construction of each tree. This out of bag (oob) data (Breiman , Random Forests, 2001) can be used for a simple estimation of the generalization error in the following way:

- For each tree all training data objects which were not used in its construction are put down that tree to get a classification.

- Each training data object is assigned the class label that got the most votes during the test run in step 1.

- The generalization error is estimated by dividing the number of training samples where the labels applied in step 2 did not match the real classes by the complete number of training samples.

### Variable importance

A measure for the variable importance can be obtained by randomly permuting the values of the Features in the dataset as follows:

- Each training data object is put down the trees where it was oob (i.e. left out in the training) and the overall number of correct votes is counted by variable c.

- The values of one feature m in the dataset are randomly permuted.

- The first step is repeated with the permuted dataset and the number of correct votes is counted by p.

- The importance of feature m is estimated by

c-pN

where N is the number of trees in the forest.

### Proximity

One of the most useful features in random forests is a proximity matrix which is an N×Nmatrix, where N is the number of training cases used in building the forest and each entry Pij is a measure for the similarity between cases i and j. The matrix entries are computed during the forest construction in the following way:

- After a tree is build, the complete training set is put down that tree and classified.

- Whenever a tree assigns the same class to training objects i and j in step 1, the value Pij is incremented by 1.

- After the complete forest is constructed, the entries Pij are normalized by the number of trees in the forest.

Several other useful features are based on the proximity matrix, such as the Prototypes or the detection of outliers.

### Class prototypes

prototypes can be calculated based on the proximity matrix by

- Identifying the training data object with the most other class c training data objects among its k nearest neighbor based on the proximity measures.

- Taking the median values for each feature from the k objects identified in step 1 and constructing a prototype object with these medians.

### Outlier measure

Random Forest defines outliers with respect to their class membership as training data objects, whose proximity to all other objects in the same class is small.

The average proximity of a an object x in class c to all a other objects in class c is

Px=classk=cPx,k2.

Based on this average proximity, a raw outlier measure for point x is defined as

OutlierRawx=NPx

Where  N is the number of samples in the training set.

The final Outlier Measure of a point x is calculated by

Outlier(x)=OutlierRawx-Median(OutlierRawxi)MAD(OutlierRawxi)

Where Median(OutlierRawxi) is the median of all raw outlier measures of objects in class class(x) and MAD(OutlierRawxi) is the Median Absolute Deviation of all raw outlier measures in class class(x).

### Unsupervised training

In case of unlabeled training data, the random forest method considers all training data to belong to class 1 and creates a synthetic dataset of the same size and dimension as the training data set, which is labeled as class 2. A synthetic data object of class 2 is created by uniformly sampling from the value distribution for each feature.

The original training data and the synthetic data can then be combined and used to construct a two-class random forest with most of the original options such as the Proximity Matrix, Outlier Detection and Variable Importance.

### Semi-supervised classification

# Experiments

Figure 9 provides an overview of the steps involved in the classification and evaluation approach of this thesis.

Fig. 9  Experimental Sequence

Data Acquisition was not part of this thesis, the roller bearing data sets available at (Case Western University) were used as benchmark. A brief overview of these datasets is given in 5.1. The preprocessing and feature extraction steps are summarized in 5.2 and chapter 5.3 describes the classification and evaluation sequences.

All experiments were completely implemented as MATLAB scripts.  Except for the Random Forest classification, all One-Class-Classifiers were realized with mappings from the Dd_tools toolbox provided by the TU Delft (Tax D. M., 2012). The Random Forest approach as outlined in 5.3 was first evaluated based on an R-package (Liaw & Wiener, 2002) and then integrated into the Dd_tools framework.

## Roller Bearing Datasets

Rotating Machines are very common in various industrial applications. In manufacturing, most machine failures are linked to bearing faults (Lou, Loparo, Discenzo, Yoo, & Twarowski, 2004). Consequently, much effort has gone into the development of classification and prediction techniques for bearing faults (Marwala, 2012), (Nelvamondo, Marwala, & Mahola, 2006), (Li, Chow, Tipsuwan, & Hung, 2000).

Roller bearings consist of two concentric rings, the inner and outer raceway (IR and OR), with a set of rolling elements running between their tracks, as illustrated in Figure 12.

Fig. 10 Roller Bearing

The rolling elements are usually contained in a cage to prevent contact between elements.

Roller bearings generate vibration signals with characteristic shapes, depending on the conditions of the raceways and the rolling elements. Faults in one of these parts typically manifest themselves in characteristic frequencies in the vibration signal (Li, Chow, Tipsuwan, & Hung, 2000).

The roller bearing data set collection provided by (Case Western University), contains vibration signals representing the following roller bearing conditions:

- Normal conditions

- Ball fault conditions

- Inner raceway fault conditions

- Outer raceway fault conditions

The individual datasets in the collection are defined by the parameters

- Position of the accelerometer for data acquisition (fan end or drive end)

- Rotation speed (1797rpm,1772rpm,1750rpm,1730rpm)

- Motor Load, correlated to the Rotation speed (0HP,1HP,2HP,3HP)

- Sample rate (12k and 48k)

In this thesis, for each condition the dataset acquired from the drive end, at a speed of 1797 and with a sample rate of 48k was chosen. Figure 3 displays sequences of each data set corresponding to the first five revolutions of the roller bearing.

Fig. 11 Roller Bearing Sample Data

To simulate a semi-supervised scenario, only the normal condition data was used in classifier training. The three fault data sets were used in the evaluation of the trained models exclusively.

## Feature Extraction

In a preprocessing step, the large data sets representing the normal and the three fault conditions were split into equal segments with a length corresponding to five roller bearing revolutions. The Kurtosis was calculated directly from each segment. For the extraction of MFD and MFCC, each segment was further split into 15 frames of equal length. Of each of these frames, 13 MFCCs and 13 HFDs were extracted.

Table 1 illustrates the schema of each of a feature sets.

|  | MFCC1 | … | MFCC13 | MFD1 | … | MFD13 | kurtosis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Feature Vector 1 | ### | ### | ### | ### | ### | ### | ### |
| … | ### | ### | ### | ### | ### | ### | ### |
| Feature Vector N | ### | ### | ### | ### | ### | ### | ### |

Table 1 Feature data set schema

### Mel Frequency Cepstral Coefficients

The MATLAB package used for extracting the MFCCs (Ellis, 2005) is part of a collection designed for feature extraction in the speech recognition domain. It is highly adaptable through a large number of parameters of which many require specific domain knowledge. For the purposes of this thesis, default settings proved to be sufficient for most of these parameters. The number of Short Time Foruier Transform frames was set to 15 and the number of MFCCs extracted from each of these frames was set to 13.

Figure 14 shows 13 MFCCs from the same frame of each dataset.

Fig. 12 Thirteen Mel Frequency Cepstral Coefficients

### Higuchi Fractal Dimensions

The Higuchi Fractal Dimension method was implemented according to the definition in (Polychronaki, et al., 2010) .  A Weierstrass function with known fractal dimension was used to verify correct behavior of the HFD-function.

Since no exact rules exist for the selection of the free parameter kmax  , the parameter was varied within a certain range, based on experimental results (Polychronaki, et al., 2010). In order to get the same number of HFDs as MFCCs, each segment was split into 15 frames and for each frame 13 different HFDs where calculated by selecting a different value for kmax each time. With this approach, the best kmax values could later be selected by PCA or similar feature selection methods.

The graph in figure 12 illustrates HFD values of the first 15 feature vectors of each set, with  kmax=6.

Fig. 13: 15 Higuchi Fractal Dimensions with k=6

The fractal dimensions of the normal signal obviously exceed the expected fractal dimensions of a curve, which should range between one and two. The reason for this may be related to the specific shape of the normal signal, which looks like random noise and shows no discernible patterns. However, what really matters in the context of feature extraction are not absolute feature values, but values which are well suited to distinguish conditions. Figure 13 suggests that HFDs are a good choice in terms of feature extraction for the given data sets.

## Random Forest Data Description

The Outlier Measure of Random Forest as introduced in 4.6 is only defined for the training data set and cannot be applied to unseen test data. It can however be used to identify critical or implausible data in the training set. Such data can then be removed or modified before a Random Forest retraining run.

Fig. 14 shows a plot of the Outlier Measures for a Random Forest trained with 427 roller bearing normal feature vectors.

Fig. 14 Outlier Measures of Roller Bearing normal training data

According to a rule of thumb given in (Breiman & Cutler, Random Forests), data points with an Outlier Measure beyond a threshold of about 10 require closer inspection. Figure 14 shows clearly, that the Outlier Measures for all training data points are smaller than 10. Consequently, no further processing of the training data set was required.

Another useful feature which provides some insight into certain aspects of training data, is the variable importance measure introduced in 4.2. Figure 15 shows the variable importance measures calculated during a random forest training with 427 normal feature samples.

Fig. 15 Attribute Importance of Roller Bearing normal features training set

The attribute importance can be defined as mean decrease in accuracy or as mean gini decrease, with both measures resulting in a different importance order of the attributes. An application of the importance measures as feature reduction method was evaluated as part of this thesis (5.5.2).

For the application of random forests in the semi-supervised outlier detection scenario assumed in this thesis, a random forest one-class classifier was implemented according to the generic approach introduced in 3. The classification sequence involves the following steps:

- Training of a random forest in unsupervised mode (4.5), with a subset of the normal feature set

- Calculation of a normal prototype pnormal  (4.4)

- Computation of the Euclidean distances dxn,pnormal between test objects xn and the normal prototype pnormal

- Calculation of a distance based threshold according to definition  (3.3)

- Classification of the test objects as defined by (3.1)

Figure 16 illustrates the Euclidean distances between the normal prototype and a test set consisting of 120 samples, as calculated during a test run of the classification approach described above.

Fig. 16 Euclidean Distances between test data and normal prototype

The plot of Euclidean distances shows, that a constant classification threshold simply defined as the highest Euclidean distance among all distances between normal samples and the normal prototype (i.e. FT-=0), was perfectly suited to separate normal and outlier roller bearing data. For robustness against outliers in the training set, the threshold should be calculated with FT->0.

## Classification and Evaluation

All experiments were conducted on an Intel(R) Core(TM)2 Duo CPU P7450 machine with a clock speed  of 2.13GHz  and 8129 MB of RAM.

Two experiments were defined to evaluate the semi-supervised outlier detection approach introduced in this thesis. In a first experiment, the classifiers were trained and tested repeatedly to estimate several performance measures. Goal of a second experiment was to evaluate the applicability of the random forest variable importance measure for feature reduction. The two experiments and the results are described below.

### Performance evaluation

One experimental sequence comprised the following steps:

- Construction of a training set with 368 objects, sampled without replacement from the normal features

- Construction of a balanced test set with 180 objects, by combining the normal objects not used for training with 30 objects sampled without replacement from each of the fault feature sets

- Training of each  classifier with the training set

- Testing of each classifier using the test set

- Taking the training and testing runtime as well as the error type I (fraction of rejected normals) and error type II (fraction of accepted outliers) for each classifier

The complete sequence was run ten times.

To compare classification results, several performance measures were calculated for each classifier (averaged over the ten runs):

- training and testing time

- type I and type II errors

- Accuracy=#True Outliers+#True Targets#True Outliers+#True Targets+#False Outliers+#False Targets

Table 4 contains a summary of the results of one complete experimental run, where FT- was set to 0.1 for all classifiers.

|  | Parameter Settings | Average Training Runtime | Average Testing Runtime | Fraction of Rejected Normals (e 1) | Fraction of Accepted Outliers (e 2) | Accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| RF-dd | #trees=100 #nodes=737 mtry=5 | 0.9432 sec | 0.0057 sec | 0.1044 | 0 | 0.9478 |
| SV-dd | Gaussian kernel sigma=6 | 20,2460 sec | 0.0093 sec | 0.5300 | 0 | 0.735 |
| K-means | errtol=10-5 k=5 | 0,019 sec | 0.005 sec | 0.1422 | 0 | 0.9289 |
| K-center | #tries=25 K=5 | 0,4289 sec | 0.446 sec | 0.1100 | 0 | 0.945 |
| N-dd | - | 0,4585 sec | 0.013 sec | 0.1278 | 0 | 0.9361 |
| Parzen | h=1.5127 | 0,2277 sec | 0.0259 sec | 1 | 0 | 0.5 |
| Som | 2D,#neurons=675 h=[0.6,0.2,0.01] η=[0.5,0.3,0.1] | 162,045 sec | 0.0058 sec | 0.1733 | 0 | 0.9133 |

Table 4 Experimental results

Figures 16 and 17 compare the time performances and accuracies of the classifiers.

Fig. 17 Time Performance

Fig. 18 Accuracies

A log-scale is used for the illustration of time performance values, since the training and testing times of the individual classifiers varied on a wide scale. The SOM-dd had by far the longest average training time, followed by SVDD.  Fastest in terms of average training time was K-Means-dd. In a typical real world application, where classifiers would be trained offline and only once, the time needed to classify a new object is probably the more important measure. Best in this category was again the K-Means-dd, K-center, whose training and testing time were approximately equal had by far the longest testing time.

Five of the seven classifiers tested achieved an accuracy score above 0.9, rf-dd had the best results with an accuracy of 0.9478. The two worst classifiers in terms of accuracy were SVDD and Parzen-dd. Parzen-dd rejected all objects of the balanced test set and thus achieved an accuracy of only 0.5. A feature scaling prior to training of the Parzen-dd did not improve performance significantly.

### Feature reduction experiment

The feature reduction experiment involved the following steps:

- Construction of a training and a test set as in the performance experiment

- Training of a random forest classifier, calculation of variable importance

- In a loop starting with the most important feature, adding the next most important feature in each iteration until the feature set is complete:

  - Training of each classifier (except random forest) using the reduced training set

  - Testing of each classifier using the reduced test set

  - Calculation of error type 1 and error type 2 for each classifier

This complete sequence was repeated several times, with a different sample training and test set at each run. The resulting averaged accuracies for each reduced feature set are illustrated in figure 19.

Fig. 19 Accuracies with reduced features

Figure 19 shows, that for most classifiers the number of features could be reduced considerably, without sacrificing accuracy. This could be an indication for redundancies in the feature set, which in turn would be the result of correlations between individual features.

The outcome of this experiment proofs, that the random forest variable importance could be used as a suitable feature reduction methods in this semi-supervised setting.

# Conclusions

# References

Banerjee, A., Chandola, V., & Kumar, V. (2007). Outlier Detection: A Survey.

Bishop, C. M. (2009). Pattern Recognition and Machine Learning. Springer.

Breiman , L. (2001). Random Forests. Machine Learning 45, S. 5-32.

Breiman, L., & Cutler, A. (kein Datum). Random Forests. Von http://stat-www.berkeley.edu/users/breiman/RandomForests/cc_home.htm abgerufen

Case Western University. (kein Datum). Case Western University Bearing Data Center. Von http://csegroups.case.edu/bearingdatacenter/pages/welcome-case-western-reserve-university-bearing-data-center-website abgerufen

Domingos, P. (October 2012). A Few Useful Things You Need To Know About Machine Learning. Communications of the ACM Volume 55 Issue 10, S. 78-87.

Ellis, D. P. (2005). PLP. RASTA,MFCC and inversion in MATLAB. Abgerufen am 2013 von http://www.ee.columbia.edu/~dpwe/resources/matlab/rastamat/

Esteller, R., Vachtsevanos, G., Echauz, J., & Litt, B. (February 2001). A Comparison of Waveform Fractal Dimension Algorithms. IEEE Transactions On Circuits And Systems-I: Fundamental Theory and Applications, Vol. 48, No.2,.

Kolerus, J., & Wassermann, J. (2008). Zustandsüberwachung von Maschinen . Wien: Expert Verlag.

Li, B., Chow, M. Y., Tipsuwan, Y., & Hung, J. C. (October 2000). Neural-Network-Based Motor Rolling Bearing Fault Diagnosis. IEEE Transactions on Industrial Electronics.

Liaw, A., & Wiener, M. (2002). Classification and Regression by randomForest. Von http://CRAN.R-project.org/doc/Rnews/ abgerufen

Lou, X., Loparo, K. A., Discenzo, F. M., Yoo, J., & Twarowski, A. (2004). A model-based technique for rolling element bearing fault detection. Mechanical Systems and Signal Processing, S. pp.1077-1095.

Mandelbrot, B. B. (2004). Fractals And Chaos. Berlin: Springer.

Marwala, T. (2012). Condition Monitoring Using Computational Intelligence Methods. Springer.

Myers, C., Japkowicz, N., & Gluck, M. (1995). A Novelty Detection approach to classification. Proceedings Of The Fourteenth International Joint Conference On Artificial Intelligence, S. 518-523.

Nelvamondo, F. V., Marwala, T., & Mahola, U. (December 2006). Early Classification of Bearing Faults using Hidden Markov Models, Gaussian Mixture Models, Mel Frequency Cepstral Coefficients and Fractals. International Journal of Innovative Computing, Information and Control.

Polychronaki, G. E., Ktonas, P. Y., Gatzonis, S., Siatouni, A., Asvestas, P. A., Tsekou, H., et al. (23. June 2010). Comparison of fractal dimension estimation algorithms for epileptic seizure onset detection. Journal Of Neural Engineering 7, S. 18pp.

Quinlan, R., & Kohavi , R. (1999). Decision Tree Discovery. Stanford.

Raghavendra, B. S., & Dutt, D. N. (2010). Computing Fractal Dimension of Signals using Multiresolution Box-counting Method. International Journal of Information and Mathematical Sciences 6:1.

Tax, D. M. (May 2012). DDtools, the Data Description Toolbox for Matlab. S. Version 1.9.1.

Tax, D. M., & Duin, R. P. (January 2004). Support Vector Data Description. Machine Learning, Volume 54, Issue 1, S. 54-66.

Tax, M. J. (2001). One Class Classification . Concept Learning in the absence of counter-examples (Ph.D. Thesis). Delft.

van der Heijden, F., Duin, R. P., de Ridder, D., & Tax, D. M. (2004). Classification, Parameter Estimation and State Estimation. 2005: Wiley.

Ypma, A., & Duin, P. W. (1998). Support Objects for Domain Approximation. Delft: Faculty of Applied Sciences .